FORCE-RISCV Support for RISC-V Trap Delegation, Trap Redirection

Introduction

Enhance FORCE-RISCV to support the RISC-V Trap Delegation feature. Include support for software Trap Redirection, as well.

This document describes the FORCE-RISCV implemtation for the Trap Delegation and the Trap Redirection features.

Trap Delegation

Support for the RISC-V hardware trap delegation, ie, the Machine-mode medeleg register, has been implemented in FORCE-RISCV*. Register fields and register field choices are in place for the medeleg register.

Heres an excerpt from the riscv/arch_data/system_registers.xml that displays a few of the medeleg register fields added:

As can be inferred from the above, each bit in the medeleg register, starting with the least significate bit, controls a single synchronous exception. When set, an exception normally taken in Machine mode is instead taken in Supervisor mode.

Also, from the riscv/arch_data/register_field_choices.xml file:

For each medeleg register field, there is a corresponding set of choices, to indicate whether or not to enable delegation, ie, set a bit in the medeleg CSR.

Existing and new test templates have been implemented that exercise trap delegation, for the (synchronous) exceptions that the simulator used by FORCE-RISCV, ie, spike, currently supports. Choice modifiers may be used at test/thread initialization time, to control which exceptions to delegate and how often.

Example test thread initialization sequence:

*note: RISC-V asynchronous (interrupt) trap delegation is supported via the mideleg register. Support for same in FORCE-RISCV is TBD.

The above discussion focuses on setting up trap delegation at test/thread initialization time. A test that starts out generating in Machine mode could of course directly read/modify/write back the medeleg CSR to enable/disable trap delegation for select exceptions, and then employ the FORCE-RISCV System Call, to switch to a subordinate privilege level, before generating exception causing instruction sequences.

Trap Redirection

Trap Redirection, as opposed to trap delegation, is performed by software. From the RISC-V Privileged spec:

Code is issued in Machine mode, as part of an exception handling sequence, to redirect the trap* from Machine mode, to a subordinate privilege level (in our case Supervisor mode) where the exception can itself be handled.

Trap Redirection could be used in conjunction with Trap Delegation. Whichever exceptions that are not directly delegated, could be redirected in software.

*How is a trap redirected? In Machine mode…
1.  Instructions are issued to copy the Machine mode exception state (CSRs mepc, mcause, mtval) to the corresponding Supervisor mode CSRs (sepc, scause, stval). 
2. The Machine mode return address (mepc CSR – actually the address of the excepting instruction) is overwritten with the Supervisor mode exception base address (stvec CSR).
3. The Supervisor mode previous privilege field of the mstatus register (mstatus.SPP) is set with the current value of the Machine mode previous privilege field (mstatus.MPP) which represents the privilege level the exception was originally taken from.
4. The Machine mode previous privilege field (mstatus.MPP) is set to 1 (Supervisor mode).
5. An MRET instruction is issued, which in effect causes control to passed to the Supervisor exception dispatcher. The exception is then processed as if the exception had been taken in Supervisor mode.

Trap Redirection

The Trap Redirection code implemented in FORCE-RISCV has been implemented as an exception handler, and can be instantiated on a per exception basis when generating the exception handlers in Machine mode.

A set of TrapRedirect choices has been defined and included in the General Choices group, to give control over which traps (exceptions) should be redirected. This choices group is queried as exception handlers are being generated in Machine mode, to determine which exceptions to redirect.

Heres an excerpt from the riscv/arch_data/general_choices.xml file, showing a subset of the Trap Redirection specific choices:

There is an entry for each synchronous exception (trap) that can be redirected.

The comprehensive handler sets (defined in py/riscv/exception_handlers/default_comprehensive_exception_handlers.json) currently includes a single entry for the trap redirect exception handler:

Existing and new test templates have been implemented that exercise trap redirection, for the (synchronous) exceptions. Choice modifiers may be used at test/thread initialization time, to control which exceptions to delegate and how often.

Example test thread initialization sequence:

Trap Delegation/Redirection Choices Modifier

For convenience, a Choices modifier has been implemented, to allow Trap Delegation and Trap Redirection to be more easily controlled.

Example test template:

Note the use of the FORCE-RISCV TrapsRedirectModifier control. As shown in the above example, an instance of the TrapsRedirectModifier control is used to specify both Trap Delegation choices and Trap Redirection choices. In the above test, approximately 50% of the time the Breakpoint exception will be delegated from Machine-mode to Supervisor mode. In the above test, approximately 50% of the time, the Environment call from U-mode and Environment call from S-mode exceptions will be redirected from Machine mode to Supervisor mode.